AIRWATCH

Rural Environmental Intelligence and Air Quality Governance Platform

# System Architecture Document

Document 04

Prepared for pre-coding planning, implementation readiness, and project evaluation.

# Architecture Diagram

ESP32 Nodes -> LoRa -> Raspberry Pi Hub -> Flask Dashboard -> SQLite -> Alerts / AI / Rankings
Local: LCD + LED + Buzzer
Remote: Website + SMS + Voice Alerts

# Layers

| Layer | Responsibility |
| --- | --- |
| Sensing Layer | MQ135 sensor reads air-quality signal |
| Edge Layer | ESP32 calculates status and triggers local alerts |
| Communication Layer | LoRa packet transmission to hub |
| Gateway Layer | Raspberry Pi receives, validates, stores data |
| Application Layer | Dashboard, maps, rankings, reports |
| AI Layer | AQI prediction and Sarvam AI language services |
| Alert Layer | LCD, LED, buzzer, SMS, voice alert |

# Packet Flow

Sensor node reads MQ135 value

ESP32 categorizes status

LCD, LED, buzzer update locally

ESP32 sends JSON packet through LoRa

Raspberry Pi receives packet and stores reading

Dashboard updates live cards and graphs

Alert engine checks threshold

GSM/Sarvam AI sends emergency notification

# Offline Operation

The ESP32 nodes work without internet. The Raspberry Pi can create a local WiFi hotspot, allowing phones to open the dashboard using a local IP address. GSM is only required for SMS delivery when cellular network is available.