AIRWATCH

Rural Environmental Intelligence and Air Quality Governance Platform

# Hardware Design Document

Document 05

Prepared for pre-coding planning, implementation readiness, and project evaluation.

# Available Hardware

| Component | Usage |
| --- | --- |
| ESP32 | Sensor node controller |
| MQ135 | Air pollution indicator |
| LoRa SX1278 TX/RX | Long-range rural communication |
| 16x2 LCD | Local AQI display |
| Red/Yellow/Green LEDs | Visual AQI status |
| Buzzer | Emergency sound alert |
| 3.7V battery | Portable node power |
| Raspberry Pi | Central hub and web server |
| GSM module | SMS alerts |

# ESP32 Pin Plan

| Device | Connection |
| --- | --- |
| MQ135 AOUT | GPIO34 |
| Green LED | GPIO12 through 220 ohm resistor |
| Yellow LED | GPIO13 through 220 ohm resistor |
| Red LED | GPIO14 through 220 ohm resistor |
| Buzzer + | GPIO27 or GPIO26 |
| LoRa SCK/MISO/MOSI | GPIO18/GPIO19/GPIO23 |
| LoRa NSS/RST/DIO0 | GPIO5/GPIO22/GPIO21 |

# Power Notes

LoRa module must use 3.3V only.

MQ135 works better on 5V/VIN, but ESP32 ADC input must remain within safe voltage.

GSM module must use external 3.7V to 4.2V supply with common ground.

LEDs require 220 ohm resistors. 220 ohm color code is red-red-brown-gold.

# Raspberry Pi Connections

| Module | Pi Pins |
| --- | --- |
| LoRa VCC/GND | 3.3V and GND |
| LoRa SPI | GPIO11 SCK, GPIO9 MISO, GPIO10 MOSI |
| LoRa NSS/RST/DIO0 | GPIO8 or GPIO7, GPIO25, GPIO24 |
| GSM TX/RX | Pi RX GPIO15, Pi TX GPIO14 with voltage divider |
| GSM Power | External battery/supply, common GND with Pi |