AIRWATCH

Rural Environmental Intelligence and Air Quality Governance Platform

# LoRa Communication Protocol

Document 10

Prepared for pre-coding planning, implementation readiness, and project evaluation.

# Purpose

LoRa enables long-range, low-power, low-cost communication from ESP32 nodes to Raspberry Pi hub without internet or SIM cards.

# Packet Format

NODE_ID|MQ135|AQI|CATEGORY|BATTERY|TIMESTAMP
Example: NODE_01|720|286|Poor|78|10:42

# Transmission Rules

Send packet every 30 to 60 seconds during demo.

Use shorter interval only for live demonstration.

Include node ID in every packet.

Retry once if ACK is not received, if ACK feature is implemented.

Use checks to ignore corrupted or incomplete packets.

# Receiver Behavior

Listen continuously

Decode packet

Validate node ID

Store reading in database

Update node last_seen time

Trigger alert if threshold crossed

# Frequency Note

Use the frequency supported by the available LoRa module. Keep transmitter and receiver frequency, bandwidth, spreading factor, and sync word identical.